21-05-2018

MECE 480 Final

Name:                        Surname:                   Number:

Q1) A discrete time system is given by the transfer function equations as follows:

Gz=4(z+2)z(z+0.5)

Use dead beat control and replace the closed loop poles to locations z1,2,3,4=0. The output signal is assumed to follow the reference unit step signal at most 4 time steps. (50 point)

Q2) A discrete system is given by the difference equation

xk+1=00.40.10xk+21uk           yk=11xk

- Find the eigenvalues (10 point)

- Is this system completely controllable (10 point)

- What is the transfer function of this system (10 point)

- Try state feedback to replace the poles of the system to locations z1=z2=0. Is this possible? Give mathematical reasoning. (10 point)

- Try state feedback to replace the poles of the system to locations z1=0 and z2=-0.2.  Is this possible? Give mathematical reasoning. (10 point)