21-05-2019

MECE 480 Final

Name:                        Surname:                          Number:                                 Signature:

Q1) A discrete time system is given by the transfer function equations as follows:

Gz=2(z+1)z(z-0.5)

Use dead beat control and replace the closed loop poles to locations z1,2,3,4=0. The output signal is assumed to follow the reference unit step signal at most 4 time steps. (50 point)

Q2) A discrete system is given by the difference equation

xk+1=0-0.90.10xk+abuk           yk=11xk

- Find the eigenvalues (10 point)

- What condition(s) should be satisfied such that system representation becomes completely controllable (12 point)

- Let’s take a=1 and b=1 and find the transfer function of this system (12 point)

- Let’s take a=1 and b=1 and try state feedback to replace the poles of the system to locations z1=z2=0. Is this possible? Show mathematically this is possible or not. (16 point)