MECE480 Homework I

Due date: 21-05-2019

Q1) A digital system is given by the formula

xk+1=-2-1-2-1xk+11uk and

- Find the transfer function of this digital system

- Find the eigenvalues of this digital system.

- Show that this system is not completely controllable.

- Even if it is not completely controllable, it is possible for this system to move both of the eigenvalues to z=0 by state feedback. Show the possibility of this process.

MECE480 Homework II

Due date: 21-05-2019

Q1) An open loop digital system is given by the open loop transfer function Gz=K(z+0.3)(z-0.3). For which K values this system becomes unstable (the closed loop poles of the system passes the unit circle outwardly).

MECE480 Homework III

Due date: 21-05-2019

Q1) An open loop digital system is given by the open loop transfer function Gz=(z+1)(z-2). Use dead beat control and replace the closed loop poles to locations z1,2,3=0. The output signal is assumed to follow the reference unit step signal at most 3 time steps. Find the dead beat controller structure C(z)

MECE480 Lab Project

Due date for the report: 30-05-2019

- Construct the dead beat controller C(z) designed in Homework III and control the system G(z) in Homework III using a negative unity feedback by SIMULINK.

- Take the reference signal as unit step input, find the error and output signals and draw them. (Let sampling time be T=1 seconds and simulation time be 10 seconds)

- Take the reference signal as unit step input, find the error and output signals and draw them. (Let sampling time be T=0.11 seconds and simulation time be 2 seconds)

- Take the reference signal as ramp input, find the error and output signals and draw them. (Let sampling time be T=1 seconds and simulation time be 10 seconds)

- Take the reference signal as ramp input, find the error and output signals and draw them. (Let sampling time be T=0.1 seconds and simulation time be 2 seconds

- In the project report explain whether your C(z) is successful or not

NOTE: Homeworks and lab project can be done in a group of at most 4 students