24-04-2019

MECE480 Midterm

Q1) The z-domain open loop transfer function of a discrete time system is given by Gz=1z-1. This system is controlled by a controller Cz=Kz+az+b. The root locus plot of H(z)=C(z)G(z) it is observed that z=-1±2  are break-in and break-away points.

- What are the values of a and b? (10 point)

- Obtain the root locus plot of H(s). (15 point)

- At which value the root locus plot passes from the imaginary axis of the z plane. (10 point)

- Find the set of equations (do not solve the equations just find the set of equations) that corresponds the intersection of the root locus plot with the unit circle. (10 point)

Q2) A continuous time system is given by the difference equations as follows:

x=-110-1x+01u,                y=10x

Let the sampling time be T=0.01 seconds and discretize the system and obtain the corresponding difference equation.

Q3) For the discrete time system below:

xk+1=0.510.75-0.5xk+10uk,                yk=01xk

- Find the discrete time transfer function. (10 point)

- Assuming that x0=00 if we apply uk=[1 -1  1 -1…….]  find yk. (10 point)

- Is this system is BIBO stable, explain. (5 point)

- Assume uk=0 however x0=ab show that finding xk, the system is stable but not asymptotically stable. (10 point)